# AECO Safety Vision – Helmet and Vest Detection

## Executive Summary

This project presents a computer vision prototype designed to detect safety equipment in construction environments. The objective is to determine whether workers are wearing helmets and safety vests using a YOLO-based object detection model. Such systems could assist safety monitoring by automatically analyzing images and identifying missing safety equipment.

The dataset used for this project was created using Roboflow and contains 57 labeled images. The dataset includes four classes: Helmet, No-Helmet, Vest, and No-Vest. The dataset split includes 41 training images, 12 validation images, and 4 test images.

## Model and Training

The model used in this project is a YOLO-based object detection architecture. The model was trained using the Ultralytics framework in Google Colab. The training configuration included an image size of 640 pixels, a batch size of 16, and a training duration of 30 epochs.

The training process took approximately eight minutes using the Google Colab CPU runtime.

## Results

The trained model achieved the following validation metrics:

Precision: 53.3%

Recall: 27.6%

mAP@50: 35.2%

These results demonstrate that the model can detect helmets and safety vests in several cases. However, the recall score indicates that the model still misses many instances of safety equipment.

## Limitations

The model performance is limited by several factors including the small dataset size, crowded construction scenes, strong lighting conditions, and selfie-style images.

## Future Improvements

Future work should focus on expanding the dataset to include more construction scenarios, diverse lighting conditions, and different camera angles.